| الأكاديمية الدولية للعلوم الصحية تحت إشراف الهيئة السعودية للتخصصات الصحية |  | International Academy for Health Sciences UNDER SUPERVISION SAUDI COUNCIL HEALTH SPECIALITIES |
| --- | --- | --- |

Subject: Dental material II

Choose the correct answer

1- An impression material is a material used to

A- Restore a tooth structure

B- Cast a model

C- Record the form of the tissue to which it is applied

D- None of the above

2- The impression material should be

A- Toxic

B- Irritant

C- Allergic

D- None of the above

3-synerisis :

a-material absorb water & shrink

b- material loose water & shrink

c- material loose water & swell

d-material absorb water & swell

4- The impression gives

A- A positive reproduction of tissue

B- A negative reproduction of tissue

C- A and b

D- None of the above

5- The alginate impression material can be classified as

A- Elastic impression material

B- Inelastic impression material

C- Gypsum impression

D- None of the above

6- One from the following is elastic impression

A- Rubber impression

B- Plaster impression

C- Compound impression

D- Zinc oxide eugenol impression

7- Plaster impression is used in

A- Dentulous jaw

B- Edentulous jaw

C- Single impression

D- Quadrant impression

8 – Hydrocolloid impression can be disinfected through

A- Soaked in the disinfection solution

B- By autoclave

C- The disinfected agent is sprayed on the exposed surface

D- Dry heat oven

9 - Elastic impression materials include

A - Agar

B - Alginate

C - Rubber impression

D - All of the above

10 - When we need a very accurate recording the fine details we use

A - Compound

B - Alginate

C - Agar

D - Rubber impression

11- If an alginate impression is stored in water the alginate

A - Will absorb additional water

B - Will expand

C - This condition is called syneresis

D – A and B

12- One of the following is the most flexible of all elastic impression materials

A - Agar

B - Polysulfides

C - Silicone rubber

D – Alginate

13- All of the following are advantages of polysulfide except

A - Low working time

B - High tear resistance

C - Easily read margins

D - Requires custom tray

14- The highest resistance impression material to tearing is

A - Zinc oxide eugenol

B - Alginate

C - Agar

D – Polysulfide

15-The setting time of zinc oxide eugenol can be accelerated by

A - Heat

B - Adding alcohols

C - High humidity

D- All of the above

16- The following properties are disadvantages of zinc oxide eugenol impression material

A - It records fine details

B - Satisfactory working and setting time

C - Compatible with the die material

D - Adhere to oral tissue, eugenol is irritant

17- Impression material can be classified into inelastic and elastic material

A- Before the time of removal from the mouth

B- At the time of removal from the mouth

C- After the time of removal from the mouth

D- None of the above

18- Impression material must have

A- Appropriate working and setting time

B- Adequate flow properties

C- Minimal dimensional change

D- All of the above

19- the setting time of impression material should be

A- Long setting time

B- Short setting

C- Appropriate setting time

D- No setting time

20- The flow of impression material should have

A- High flow properties

B- Adequate flow properties

C- Less flow properties

D- None of the above

21- The following impression material can not be used in undercut

A- Alginate

B- Plaster

C- Agar

D- Rubber

22- One of the following impression material is not used for quadrant impression

A - Alginate

B - Polysulfide

C - Silicone rubber

D - Plaster

23- All of the following impression materials need special tray  ( custom tray ) except

A - Plaster

B - Alginate

C - Polysulfide

D - Zin oxide eugenol

24- Compound impression material can be used for

A - Full jaw impression

B - Impression trays (  primary impression )

C - Peripheral sealing

D - All of the above

25- The following impression material are dimensionally accuracy except

A - Plaster

B - Zinc oxide eugenol

C - Alginate

D - Compound

26- All of the following are disadvantages of polysulfide except

A - High tear resistance

B - Require custom tray

C - Hydrophobic

D – Messy

27- The highest resistance impression material to tearing is

A - Zinc oxide eugenol

B - Alginate

C - Agar

D - Polysulfide

28 - From the disadvantages of polyether

A - Low setting

B - Accuracy

C - Stiff

D - Good stability

29- All of the following are disadvantages of polysulfide except

A - High tear resistance

B - Require custom tray

C - Hydrophobic

D - Messy

30 – All of the following are the advantages of polyether impression material except

A – Stiff

B – Fast Setting

C – Clean

D – Delay pouring

31 - All of the above of the following impression material have the ability to record fine details of oral tissue except

A – alginate

B – Zinc oxide eugenol

C – Compound

D – Agar

32 - The following impression material are easily read margins except

A – Polysulfide

B – Silicone rubber

C – Polyether

D – Compound

33 - The following impression materials do not need a separating agent before pouring except

A – Plaster

B – Alginate

C – compound

D – Zinc oxide eugenol

34 - The following impression material is extremely high accuracy

A – Compound

B – Alginate

C – Zinc-oxide eugenol

D – Silicone rubber

35 – Dimensional accuracy means

A – The ability to maintain fine details

B – The ability to record fine details

C – Easy to be removed from the mouth

D – All of the above

36 – biological compatibility of the impression material involves

A – Freedom from toxic or Irritant constituents

B – Pleasant odour

C – good esthetic

D – All of the above

37 - The following impression material are dimensionally stable except

A – Alginate

B – Zinc oxide eugenal

C – Plaster

D – Silicone rubber

38 - We can delay the pouring of these impression material except

A – Plaster

B – Silicone rubber

C – Polyether

D – Alginate

39 - Rubber impression include

A – Polysulfide

B – Silicone

C – Polyether

D – All of the above

40 – Deformation of a metal or alloy due to high temperature, is termed

A – Corrosion

B – Ductility

C – Malleability

D – Sag

41 – Alloys must be

A - Soft

B - Hard enough to resist wear

C - Not too hard

D - All of the above

42 – Proportional limit is

A – The rigidity or stiffness

B – The amount of stress required to produce permanent deformation of a material

C – The amount of stress required to produce change in length

D – Non of the above

43-To prevent oxidation of the gold alloy during casting we use

A – The cooling system

B – Flux

C – Borax

D – Non of the above

44– The degree to which a material can be permanently deformed by a tensile force without under going fracture is termed

A – Malleability

B – Ductility

C – Proportional limit

D – Sag

45 – The degree to which a material can be deformed by compressive force without undergoing fracture

A – Corrosion

B – Malleability

C – Ductility

D – Modulus of elasticity

46 – All of the following are the advantages of amalgam restoration except

A – High strength

B – Easy clinical handling

C – Low cost

D – Lack of adhesive properties and non-aesthetic character

47 – All of the following are the disadvantages of amalgam restoration except

A – Lack of adhesive to tooth structure

B – Non-aesthetic

C – High strength

D – Non of the above

48 – Investment is used

A – For taking as impression material

B – For casting

C – For cementation

D – For restoration

49 – All of the following are the requirements of an ideal investment except

A – It should be easily manipulated

B – It should produce a smooth surface

C – It must decompose at high temperature

D – It must be porous

50 - Tooth colored restorative materials include the following except

A – Glass inomer cement

B – Composite

C – Compomers

D – Amalgam

51 - A very common error in spruing process, is the use of

A – Too long sprue

B – Too thin sprue

C – A and B

D – None of the above

52 – When two or more metals are mutually soluble in each other, this is termed

A – Liquidus

B – Solidus

C – Alloy

D – Non of the above

53 – The temperature below which a metal is  completely solid (i.e the temp at which it begins to melt)

A – SoLidus

B – Liquidus

C – Alloy

D – Non of the above

53 – Corrosion is important for dental alloy because corrosion can lead to

A – Smoothing of the surfaces

B – Strengthen of the restoration

C – Liberation of elements from the alloy can produce discoloration of solt tissues and allergic reaction

D – None of the above

54 – Dental alloys can be classified into

A – Gold based alloys

B – Metal-ceramic alloys

C – Alloys for removable partial dentures

D – all of the above

55 – One of the following dental cement is fluoride release

A – Glass inomer cement

B – Zinc-phosphate cement

C – Resin cement

D – Zinc oxide eugenol

56– The production of a porcelain jacket crown involves

A – Compaction

B – Firing

C – Glazing

D – All of the above

56.The positive reproduction of the oral tissues can be obtained by pouring ------------------------------ into the impression.

- Dental stone.

- Alginate.

- Zn O/E.

- None of the above.

57.Bright shiny casting with rounded margins is a result of --------------------------- of the wax pattern.

- Broken investment.

- Hardening.

- Incomplete elimination.

- Additives.

58.------------------------------- constitutes 30-55 % of the investment.

- Refractory material.

- Binder material.

- Modifiers material.

- All of the above.

59.Flux should be applied to gold when it begins to take on ---------------------------------- color.

- A red.

- An orange.

- A green.

- A white.

60.Compound impression material is considered to be ----------------------------------.

- Thermoplastic material.

- Inelastic material.

- Not dimensionally accurate material.

- All of the above.

61.Asbestos liner should be used with the investment to allow for free ----------------------------.

- Shrinkage.

- Expansion.

- All of the above.

- None of the above.

62.Silicons and polyethers can be used as -------------------------.

- Cast materials.

- Reversible materials.

- Duplicating materials.

- Inelastic materials.

62.Zinc in an alloy acts as ----------------------------.

- Scavenger.

- Decrease corrosion.

- Change shape.

- No effect.

63.Polysulfide rubber is the most ----------------------- of all rubbers.

- Accurate.

- Flexible.

- Elastic.

- Expensive.

64-Setting of glass ionomer cement is a result of an -----------------.

- Acid base reaction.

- Light cure.

- Heat cure.

- None of the above.

65.Plaster impression must be treated with ---------------------- before pouring the cast.

- Coloring agent.

- Softening agent.

- Separating agent.

- All of the above.

66.------------------------ have the characteristics of fluoride ion release.

- Zinc phosphate cement.

- Glass ionomer.

- Compomers.

- Polycharbohydrates.

67.The most accurate rubber impression material on storage is -----------------------------.

- Polysulfide.

- Condensation silicons.

- Addition silicons.

- Polyether.

68.The strongly oxidizing cone of the blowpipe flame is the ---------------------------------------------.

- Inner dark blue cone.

- Middle cone.

- Outer dark blue sheath.

- Middle purple cone.

69.-------------------------- is a liquid metal.

- Gallium.

- Iron.

- Mercury.

- Hydrogen.

70.Porcelain powder is mixed with ----------------------- to produce a paste.

- Water.

- Oil.

- Alcohol.

- Eugenol.

71.----------------------- is the deformation of a metal or alloy during porcelain firing.

- Liquidus.

- Sag.

- Grain.

- None of the above.

72.Porcelain contains quartz which acts as ----------------------Activator.

- Coupling agent.

- Softening agent.

- Strengthening agent.

73.------------------------ are added to an alloy to reduce corrosion.

- Base metals.

- Grain refiner.

- Nobel metals.

- Iron.

74.Current system of classification of dental alloys is based on physical properties and ------------------------.

- Chemical properties.

- Composition.

- Uses.

- Melting temperature.

75.Sprue diameter must be equivalent to the ----------------------- portion of the wax pattern.

- Thickest.

- Thinnest.

- Smallest.

- Margins.

76.Restorations that are used in areas of esthetic demands are usually fabricated from -------------------------.

- Gold.

- Base metal alloy.

- Cobalt.

- Porcelain.

77.----------------------- is added to resin to prevent it from self polymerization.

- Activator.

- Initiator.

- Inhibitor.

- Coupling agent.

78.Quartz filler of macrofilled resin composites is -------------------.

- Radiopaque.

- Radiolucent.

- Not affected by X-ray.

- All the above.

79.------------------------------ is the main advantage of glass ionomer cement.

- Chemical adhesion to enamel and dentin.

- Low cost.

- High wear resistance.

- Easily applied.

80.The pure alumina core porcelain has the advantage of -------------------------------.

- Increased strength.

- Superior translucency.

- All the above.

- None of the above.

81.Ceramic veneers are better than full crowns because they have good ------------------------------ with minimal tooth reduction.

- Strength.

- Bond.

- Esthetics.

- Durability.

82.Compomers and resin cements have the disadvantage of being ---------------------------.

- Time consuming.

- Poor esthetics.

- A & B are correct.

- None of the above.

83.Alumina reinforced porcelain consists of a ------------------- containing 40-50 % alumina.

- Filler.

- Quartz.

- Feldspathic glass.

- Resin.

84.Liberation of elements from a corroded metal will cause ------------------ of adjacent soft tissues.

- Discoloration.

- Allergic reaction.

- All the above.

- None of the above.