General revision

DENTAL RADIOLGY

1- Quality of a radiograph is decided by

- Density

- Contrast

- Sharpness

- All of the above

2- Size of the x-ray beam may be reduced by

a- Aluminum filter

b- Tungsten tube

c- Lead diaphragm

d- All of the above

3- X-radiation is a form of electromagnetic radiation that have neither mass nor charge and travel in wave form at a speed of :

a- 186,000miles/sec

b- 18,600,000miles/sec

c- 320,000 miles/sec

d- 32,000,000 miles/sec

4- X-rays are generated when a stream of electrons traveling from one side of an x-ray tube is suddenly stopped by its impact on the

a- Filament cup

b- Tungsten target of the anode

c- Tungsten target of the cathode

d- None-of the above

5- The x-rays with high penetration qualities have a:

a- Low kilovoltage

b- Short wavelength

c- Low frequency

d- Long wavelength

6- Image sharpness is increased by:

a- Decreasing focal spot size

b- Increasing focal spot size

c- Decreasing source film distance

d- Increasing object-film distance

7- filters are used in radiography

a- To reduce patient radiation

b- To reduce short wavelength rays

c- To increase secondary radiation

d- To increase image contrast

8- Which of the following x-ray films requires lesser exposure time as compared to rest of them?

a- B speed

b- C speed

c- D speed

d- E speed

9- The base of an x-ray film is composed of:

a- Celluloid acetate

b- Gelatin

c- Nylon

d- Bromide

10- The diameter of x-ray beam at the patient's face must not exceed

a- 12cm

b- 10cm

c- 9cm

d- 7cm

11- Intensifying screens are used with extraoral radiographic films to:

a- Increase Kvp

b- Increase exposure time

c- Improve image quality

d- Decrease radiation to the patient

12- Contrast of a radiograph mainly depends on:

a- Kvp

b- Exposure time

c- Amount of radiation

d- Milliamper

13- Sharpness of a radiograph pertains to:

a- Blackness of a radiograph

b- Difference in the blackness of two areas on the radiograph

c- Recording the boundaries of various structures on the   radiograph

d- All of the above

14- Density of a radiograph may be increased by increasing the

a- Millimpere

b- Kiloltage

c- Exposure time

d- All of the above

15- Grids are used to:

a- Reduce patient exposure

b- Reduce film fogging

c- Reduce the object-film distance

d- None- of the above

16- Fogging on the radiograph is caused by:

a- Excessive temperature of the storage area

b- Overdevelopment

c- Scattered radiation

- All of the above

17- A grid is placed:

a- Perpendicular to the film

b- Between the film and the object

c- Between the object and the source

d- On the tube head

18- Quality of a radiograph is decided by

a- Density

b- Contrast

c- Sharpness

d- All of the above

19- X-ray film is not sensitive to:

a- Blue light

b- Yellow light

c- Red light

d- b & c

20- A dark film indicated that it has been:

a- Overdeveloped

b- Underdeveloped

c- Exposed to light

d- Overfixed

21-  Latent image is a term used for

a- Under developed image on the film

b- Over developed image on the film

c- Undeveloped image before processing

d- None-of the above

22-  A processed film is to be rinsed in water to get rid of the chemical for about:

a- 10 minutes

b- 20 seconds

c- 1 minute

d- 2 minutes

23-  A foreshortened image on the radiograph is produced when the radiations are:

a- Perpendicular to the film but not to the object

b- Perpendicular to the object but not to the film

c- Perpendicular to the object and to the film

d- Parallel to the object and the film

24-……………...is used to examine the entire crown, root, and surrounding bone.

a- Periapical film

b- Occlusal film

c- Bite-wing film

d- All of the above

25- Overexposure results in:

a- Fogging

b- Light film

c- Dark film

d- Less contrast

26- Distortion of the image is:

a- Less in paralleling technique

b- Less in bisecting technique

c- Same in both technique

d- Reduced by increasing vertical angulation

27-The function of rinsing after development is to:

a- Remove the soluble chemicals from surface of film.

b- Stop the developing reaction.

c- Neutralize the alkalinity of the developer.

d- All of the above

28- All of the following are disadvantages of the automatic processor except:

a- Standarization of results

b- High expenses

c- The need for regular maintenance

d- Most machines need to process a certain minimum amount of film/day

29- The fixing agent in the radiographic fixer is the:

a- Sodium sulfite

b- Sodium thiosulfate

c- Acetic acid

d- Sodium carbonate

30- ............intra-oral film used for anterior teeth in adults as periapical or bitewing

a- Size o

b- Size 1

c- Size 2

d- Size 3

31- Beam size of the radiation is decreased by:

- Using intensifying screens

- Collimation

- Increasing the target object distance

- Filtration

32- The X-rays with high penetration qualities have a:

- Low kilovoltage

- Short wavelength

- Low frequency

- Long wavelength

33-  Sharpness of a radiograph pertains to:

- Blackness of a radiograph

- Difference in the blackness of two areas on the radiograph

- Record the boundaries of various structures on the radiograph

- None-of the above

34-  Contrast of a radiograph mainly depends on:

- Kvp

- Exposure time

- Amount of radiation

- Milliamper

35-  The penetrability of the x-rays is increased by increasing

a- Exposure time

b- Milliamperage

c- Kilovoltage

d- Collimation

36-  The base of an x-ray film is composed of:

- Celluloid acetate

- Gelatin

- Nylon

- Bromide

37-  Overexposure results in:

- Fogging

- Light film

- Dark film

- Less contrast

38- Grids are used to

- Reduce patient exposure

- Reduce film fogging

- Reduce the object-film distance

- None-of the above

39- …..................intra oral film is used to examine large areas in upper or lower jaw.

- Size 1

- Size 2

- Size 3

- Size 4

40- The efficiency of X-ray penetration is related to

a- The frequency and the wave length of the X-ray.

b- The thickness of the material through the X-ray pass.

c- The density of the material through the X-ray pass.

d-All of the above.

41- Fogging on the radiograph is caused by

a- Excessive temperature of the storage area

b- Overdevelopment

c- Scattered radiation

d- All of the above

42- ………………..is the source of electrons for the production of X-ray.

a-The cathode

b-The anode

c-The filter

d-All of the above.

43- Filter of X-ray tube is made of

a- Aluminum.

b- Copper.

c- Lead.

d- Glass.

44- The diameter of the X-ray beam at the patient face should be

a- Greater than 7cm.

b- Greater than 10cm

c- Not greater than 7cm.

d- Divergent.

45- What of them produce excess radiation

a- Round collimator

b- Rectangular collimator

c- a & b

d- Non of the above

46- Function of filter is

a- Improve quality of the beam.

b- Making the beam homogenous.

c- Improve image quality.

d- All of the above.

47- What is preferred?

a- Open end cone

b- Pointed end cone.

c- all of the above.

d-  None of the above.

48- The black areas that appear on the radiograph are called

a- Radiolucent

b- Radiopaque

c-  Radiolucent & radiopaque.

d-  None of the above.

49-……………….is used in parallel technique

a- Short cone 8 inches

b- Long cone 16 inches

c- all of the above.

d- None of the above.

50- The more the object density …………………………

a- The less the dense of the produced image.

b- The more the dense of the produced image.

c-  None of the above.

d-  all of the above.

51- The radiographic contrast is affected by object

a- Thickness

b- Density

c- Atomic number

d- All of above.

52- The more the penetration of the X-ray beam to the object………………………..will be.

a- The more the brightness of the image

b- The more the darkness of the image

c- The more the thickness.

d- The more the density.

53- ………………….is the ability to define images of small separate structures that are very close together.

a-Image resolution

b-filtration

c-Sharpness

d-Image contrast

54-  Scattering may be due to :

a-Use of high Kvp in the process of X-ray production.

b-Absence of lead – packing of the film.

c-Decrease in collimation.

d-All of the above.

55- The crown, root and surrounding bone can be examined by

a- Periapical film.

b- Bitewing film.

c- Occlusal film.

d- Non- of the above.

56- …………………..is the amount of radiation required to produce a radiograph of standard density.

a- Film size

b- Film speed.

c- Film shape.

d- None of the above.

57- A slow film require ……...............to produce an acceptable image.

a- low exposure

b- Long exposure

c- a & b

d-  None of the above.

58- Film speed is controlled by

a- Emulsion coating.

b- Size of the Ag halide crystal.

c- Shape and thickness of the crystal.

d- All of the above.

59- Films are adversely affected by

a- Heat

b- Humidity

c- Radiation

d-All of the above.

60- One of the advantage of the automatic processor is

a- Rapidity and ease of use

b- Standarization of result.

c- Elmination of the need to the dark room and processing tank.

d- All of above.

61-  X-radiation are a form of electromagnetic radiation that have neither mass nor charge and travel in wave form at a speed of :

a- 186,000miles/sec

b- 18,600,000miles/sec

c- 320,000 miles/sec

d- 32,000,000 miles/sec

62- X-rays are generated when a stream of electrons traveling from one side of an x-ray tube is suddenly stopped by its impact on the

a- Filament cup

b- Tungsten target of the anode

c- Tungsten target of the cathode

d- None-of the above

63- The penetrability of the x-rays is increased by increasing

a- Exposure time

b- Milliamperage

c- Kilovoltage

d- Collimation

64- Size of the x-ray beam may be reduced by

a- Aluminum filter

b- Tungsten tube

c- Lead diaphragm

d- All of the above

65- The x-rays with high penetration qualities have a:

a- Low kilovoltage

b- Short wavelength

c- Low frequency

d- Long wavelength

66- Image sharpness is increased by:

a- Decreasing focal spot size

b- Increasing focal spot size

c- Decreasing source film distance

d- Increasing object-film distance

67- filters are used in radiography

a- To reduce patient radiation

b- To reduce short wavelength rays

c- To increase secondary radiation

d- To increase image contrast

68- Which of the following x-ray films requires lesser exposure time as compared to rest of them?

a- B speed

b- C speed

c- D speed

d- E speed

69- The base of an x-ray film is composed of:

a- Celluloid acetate

b- Gelatin

c- Nylon

d- Bromide

70- The diameter of x-ray beam at the patient's face must not exceed

a- 12cm

b- 10cm

c- 9cm

d- 7cm

71- Quality of a radiograph is decided by

a- Density

b- Contrast

c- Shrapness

d- All of the above

72- Contrast of a radiograph mainly depends on:

a- Kvp

b- Exposure time

c- Amount of radiation

d- Milliamper

73- Sharpness of a radiograph pertains to:

a- Blackness of a radiograph

b- Difference in the blackness of two areas on the radiograph

c- Recording the boundaries of various structures on the radiograph

d- All of the above

74- Density of a radiograph may be increased by increasing the

a- Millimpere

b- Kiloltage

c- Exposure time

d- All of the above

75- Grids are used to:

a- Reduce patient exposure

b- Reduce film fogging

c- Reduce the object-film distance

d- None- of the above

76- Function of filter is

a- Improve quality of the beam.

b- Making the beam homogenous.

c- Improve image quality.

d- All of the above.

77- What is preferred?

a-Open end cone

b-Pointed end cone.

c- all of the above.

d- None of the above.

78- The black areas that appear on the radiograph are called

a- Radiolucent

b- Radiopaque

c-  Radiolucent & radiopaque.

d-  None of the above.

79-……………….is used in parallel technique

a- Short cone 8 inches

b- Long cone 16 inches

c- all of the above.

d- None of the above.

80- The more the object density …………………………

a- The less the dense the produced image.

b- The more the dense the produced image.

c- The more the thickness.

d- The more the density.

81- The radiographic contrast is affected by object

a- Thickness

b- Density

c- Atomic number

d- All of above.

82- The more the penetration of the X-ray beam to the object………………………..will be.

a- The more the brighter the image

b-The more the darkness of the image

c- The more the thickness.

d- The more the density.

83- ………………….is the ability to define images of small separate structures that are very close together.

a-Image resolution

b-filtration

c-Sharpness

d-Image contrast

84-  Scattering may be due to

a-Use of high Kvp in the process of X-ray production.

b-Absence of lead – packing of  the film.

c-Decrease in collimation.

d-All of the above.

85- The crown ,root and surrounding bone can be examined by

a- Periapical film.

b- Bitewing film.

c- Occlusal film.

d- Non- of the above.

86- Film speed is controlled by

a- Emulsion coating.

b- Size of the Ag halide crystal.

c- Shape and thickness of the crystal.

d- All of the above.

87- Films are adversely affected by

a- Heat

b- Humidity

c- Radiation

d-All of the above.

88- One of the advantage of the automatic processor is

a- Rapidity and ease of use

b- Standarization of result.

c- Elmination of the need to the dark room and processing tank.

d- All of above.

89- X-ray was discovered by professer ……………………

a- Wilhem Conrad Roentgen

b- Otto Walkhoff

c-  William Herbert Rollins

d-  Raper led

  - The electrical charge of protons is ……………

- Negative

- Positive

- Neutral

- Non of the above.

  - Electrons are maintained in their shells by ……

- Photon energy

- Binding energy

- Adhesion

- All of the above

  - ………………………to remove Q shell electron.

- A great amount of energy is needed

- Less amount of energy is needed

- No energy is needed

- None of the above

  - If the transferred energy is not great enough to

remove an orbital electron.

- Excitation occurs

- Ionization occurs

- Attraction occurs

- Absorption occurs

  - Particulate radiation is like …………………….

- Protons

- Neutrons

- X-rays

- a & b

95- The efficiency of X-ray penetration is related to

a- The frequency and the wave length of the X-ray.

b- The thickness of the material through the X-ray pass.

c- The density of the material through the X-ray pass.

d-All of the above.

96- Electromagnetic radiation has …………………

- Different energies

- Different wave lengths

- The same wave lengths

- a & b

  - Wave length is the distance between …………..

- The crest of one wave  and the crest of the next

- The crest of one wave  and the bottom of the next

- The bottom of one wave  and the bottom of the next

- None of the above

  - X-ray machine consists of ………………………

- Tube head

- Extension arms

- Control panel

- All of the above

  - ……………. is the source of electrons for the production

of X-ray.

a- The cathode

b- The anode

c-  The filter

d- All of the above

  - Filter of X-ray tube is made of …………………………

- Aluminum

- Copper

- Lead

- Glass

THE END