International Academy for Health Science

Department :Dental Assistant                                                                 Level : 3

Subject: Dental Radiology II

Student Name:………………………………………….                          Period :Morning

……………………………………………………………………………….........................................................

I- Choose The Correct Answer:(30 marks)

1- Following an exposure to radiation the residual biologic damage that remains is what type of effect?

a- Direct

b- Indirect

c- Tolerance

d- Cumulative

2- Diameter of X-RAY beam must not excced :

a- 1.50 inches

b- 2.50 inches

c- 2.75 inches

d- 1.75 inches

3- Diaphragm which is used to limit the diameter of X- ray beam is made up of :

a- Aluminum

b- Lead

c- Carbon

d- Silver

4- The X-ray beam is filterd to

a- Soften the beam

b- Increase patient exposure

c- Remove long-wave photons

d- Remove short-wave photons

5- Which of the following X-ray films require lesser exposure time as compared to rest of them?

a- B speed

b- C speed

c- D speed

d- E speed

6- What is preferred ?

a- Short cone ( 8 inches)

b- Long cone ( 16 inches)

7- The distance between the source of X-ray and the operator should be at least :

a- 14 feet

b- 12 feet

c- 6 feet

d- 3 feet

8- Preapical radiograph is useful in diagnosing:

a- Preapical infection

b- Preapical bony lesions

c- Impaction

d- All of the above

9- Paralleling radiographic technique dose the following except

a- No special equipments required such as long cone and film holder

b- Reduce magnification

c- No superimposition

d- Reduce distoration

10- In bisecting angle technique

a- The film is placed as close to the tooth as possible

b-  The film is supported parallel to the long axis of the tooth.

c- The patient should sit in upright position

d- a & c

11- The vertical  angulation ( cone position) for upper incisors

a- +ve 40 – 50

b- -ve 45 – 50

c- + ve 35 – 40

d- -ve 35 – 40

12- Elongation or shortening of the image on radiograph can be controlled by

a- Altering the vertical angulation

b- Altering the horizontal angulation

c- Altering the source-film distance

d- None of the above

13- Bite-wing radiography are useful in diagnosing all the following except

a- Incipient interproximal caries

b- periapical infection

c- Overhanging restoration

d- Interproximal calculus deposits

14- Best X-ray view for TMJ is

a- Lateral skull

b- Lateral oblique

c- trans pharyngeal

d- Panoramic

15- Which radiograph would give the best view of fracture of zygomatic arches

a- PNS view

b- Submento vertex

c- Lateral oblique mandible

d- Transpharyngeal view

16- The radiographic view taken with the patient chin placed on the cassette is :

a- P-A view

b- Towne's view

c- Water's view

d- Submentovertex view

17- Maxillary sinus is best viewed on

a- P-A view

b- Towne's view

c- Water's view

d- Submentovertex view

18- Subcondylar fractures are best viewed on

a- P-A view

b- Revers Towne's view

c- Water's view

d- Panoramic view

19- Overexposure result in

a- Fogging

b- Light film

c- Dark film

d- Less contrast

20- Radiographic film may appear too light because of using

a- A concentrated developer solution

b- Overexposure of film

c- Depleted developer solution

d- Incorrect safe light

21- Yellowish brown discoloration in X-ray is due to

a- Low temperature

b- High temperature

c- Insufficient washing

d- Exposure to light

22- Fogging on the radiograph is caused by

a- Increase in humidity

b- Improper safe lighting

c- Overdevelopment

d- All of the above

23- Film speed or sensitivity is determined by

a- Size of silver halide crystals

b- Thickness of the emulsion

c- Presence of radiosensitive dyes.

d- All of the above

24- One of the following can differentiate soft tissues of different densities

a- CT scan

b- Panoramic radiograpy

c- Submentovertex view

d- None- of the above

25- Blurred image is due to

a- High temperature

b- Increasing KVP

c- Overexposure

d-  Patient Movement during exposure

26- The protection from x- ray radiation should include the

a- Patient

b- Assistant

c- Operator

d- All of the above

27- ......................is the measurement of ionization

  - Rad

  - Rem

  - Roentgen

  - All of the above

28- Which of the following cones produces large amount of scattered during exposure as compared to rest of them .

a- pointed closed end plastic cone

b- open ended cylindrical plastic cone

c- open ended rectangular cone

d- all of the above produces the same amount

29- Most effective method of decreasing radiation is

a- E speed film

b- Decreasing exposure time

c- Decreasing kilovltage

d- Decreasing milliamperage

30- Panoramic radiographs are useful in assessment of

a- Impacted teeth

b- Both jaws prior to implant

c- Both jaws prior to planning for dentures

d- All of the above

II- Complete The Following (10 marks)

- ..rad.................is the unit of absobed dose of radiation.

- The amount of radiation produced by a dental machine and measured at theee end of cone is called....the output of the machine......................

- ....rectangular..................is most effective in reducing patients exposure from the other

cone types.

- The vertical angulation(cone position) for lower premolar is....10................

- Excessive decrease in the vertical angulation will result in image.elongation...............

- In bisecting angle technique the central rays must be directed

Perpendicular to..bisector line........................

- ......panoramic......................radiograph includes coverage of the entire maxilla and

mandible.

- Paralleling technique also called...right angle technique...........................

- Salivary gland stones are viewed on...occlusal film...........................

- Internal derangement of TMJ can be viewed by..MRI...........................

III- Write Whether The Following Statements Are True Or False (10 marks)

1-Fast film preferable more than the slow film.T

2-The operator should hOld a film in place inside the patient mouth during exposure.F

3- The length of the short cone is 16 inch. F

4-Using x-ray machines of high kvp reduces the exposure time.T

5-Delayed timer allows the operator to get out of the field of radiation during exposurTe.

6- The operatorshould not stand in contact to the patient during exposure.T

7- MRI is invasive methods used to visualize soft tissues.F

8- Excessive fixation will result in light radiograph.T

9- Film contaminated with fixer before processing result in dark spots

or lines on radiograph .F

- Somatic effects of radiation are dependent on the rate at which the

Radiation are given. T

IV- Enumrate five uses for periapical radiograph ? ( 5 marks)

AZMI